A Study on the Prediction of Extreme Water Waves at the North of Jakarta’s Coast by Weibull Statistical Distribution

Yoyok Setyo Hadiwidodoa and Jamhari Hidayat Bin Mustofab

a) Senior Lecturer, Department of Ocean Engineering, Institute of Technology, Surabaya, Indonesia

b) Undergraduate Student, Department of Ocean Engineering, Institute of Technology, Surabaya, Indonesia

*Corresponding author: yoyoksetyo@oe.its.ac.id

Paper History

Received: xxxxx

Received in revised form: xxxxx

Accepted: xxxxxxx

ABSTRACT

Prediction of the intensity of extreme waves is one of the importance of information about ocean waves especially for the safety of a variety activities on offshore, on an annual basis, with reference to relatively limited wave data based on data in the period 2004-2014 at the north of Jakarta’s coast obtained by BMKG. By using a Weibull statistical distribution function, the study shows that the maximum wave height of the 10-year period, 50-year period, and 100-year period are 1.89 meters, 2.15 meter, and 2.23 meters. Also, by using the same pattern it can be obtained that the peak wave period of the 10-year period, 50-year period, and 100-year period are 9.33 seconds, 9.51 seconds, and 9.58 seconds.

KEY WORDS: Extreme waves, peak wave period, wave height, Weibull distribution

ACKNOWLEDGEMENTS

The author would like to thank Mr. Yoyok Setyo Hadiwidodo, ST, MT, Ph.D. and Mr. Suntoyo, ST, M. Eng., Ph.D. for his guidance over the years. Also a big appreciation also to the two colleagues to help the author in this research, namely Dimas Maulana Rachman and Nuraksatra Nadhif.

REFERENCE

- Djatmiko, B. E., Perilaku dan Operabilitas Bangunan Laut di Atas Gelombang Acak. Surabaya: ITS Press, 2012

- Almar-Naes, A., Fatigue Handbook: Offshore Steel Structures. Norway: Tapir Academic Press, 1985

- Weibull, W. A Statistical Distribution Function of Wide Applicability, Journal of Applied Mechanics, Transactions of ASME, Vol. 18, pp. 293-297, 1951

- Ochi, M. K., Principles of Extreme Value Statistics and their Application, Proceedings of Extreme Loads Response Symposium, Arlington, VA, USA, Oct, 1981